Insert formulas based on which formula was defined first if a formula is used in another formula

PowerWatt=Supply VoltageVolts×Current(A)

Formulas & Metric Descriptions

CurrentA=Power(Watt)Supply Voltage(Volts)

Represents the total amount of electrical energy the battery is able to output, with respect to hours. This metric will be used to compute a 30% power design margin that the system power load must remain under.

Nominal Volatage Volts×Capacity(Amp hour)

Unit: Watt hour

Energy Rating

Discharge Capacity C-rating×Capacity(Amp hour)

Max Continuous Discharge Current

Unit: Ampere

Represents the maximum amount electrical current that can be outputted by the battery continuously. This continuous current ceiling must be greater than the system peak current load.

This metric represents the percentage of current capacity(total usable current) an electrical device will draw in an idle, nominal, or peak active state. This metric will be used to calculate the current of each electrical device at each operating state using the formula below:

Energy Rating Watt hour×0.70System Power Load (Watt)

Unit: hours

Usable Runtime Margin

UtilizationPercentage=Estimated Operating LoadAMaximum Current A×100

This metric will be used to calculate the budgeted system power load. The budgeted system power load conforms to the 30% power design margin we have selected. All budgeted operating load’s for each electrical device’s operating state will have a 30% percentage increase to the corresponding Estimated Operating Load.

Budgeted Operating Load A=Estimated Operating Load A×(1+Power Design Margin(Percentage)100)

Unit: Ampere

Budgeted Operating Load

The predicted utilization behavior for each electrical device during each utilization percentage is found below:

Estimated Operating Load A=Maximum Current A×(UtilizationPercentage100)

Unit: Percentage

Utilization

| Electrical Device | Idle Utilization | Nominal Utilization | Peak Utilization |
| --- | --- | --- | --- |
| Thruster | A given thrusters Electronic Speed Controller (ESC) is only drawing quiescent current | Thruster is cruising and opposing small currents, or used to make minor corrections to the submarine’s attitude | The thruster makes major corrections to the submarine’s attitude and opposes large currents. |
| Oak D Lite | The camera draws quiescent current and the current necessary to power, process initialization, power on the Vision Processing Unit (VPU), power on the sensory array | The Vison Processing Unit (VPU) preforms the workloads below at a less then peak utilization Frame Per Second (FPS) rate: - Base consumption and Camera streaming - AI subsystem utilization - Stereo depth pipeline subsystem - Video encoder subsystem | The Vision Processing Unit (VPU) preforms the aforementioned workloads at the maximum required Frame Per Second (FPS) rate |

Represents the power an electrical device will draw at each operating state. This metric will be used to calculate the system power load metric.

Supply Voltage Volts×Operating Load (A)

Unit: Watt

Power Idle/Nominal/Peak

Acknowledgments

All fuse current ratings are calculated via:

We acknowledge that this yields a current rating that is less than our budgeted current rating which is calculated via:

Once electrical devices are able to be bench tested we will recalculate our fuses’ current ratings to more closely align them with a practical budgeted Peak Operating current, that is in reference to our 30% power design margin.

Budgeted Peak Operating Current (A)0.75

Estimated Peak Operating Current (A)0.75